WATER POLO

POOL CERTIFICATION

ORGANIZING BODY

BUILDING NAME

OWNER

CONTRACTOR

DESIGN TEAM

DATE

WORLD AQUATICS

APPROVED BY

DATE

SIGNATURE

In force as from 1 April 2026

Table of Contents

DRAWING SUBMITTAL REQUIREMENTS	3

MEASUREMENT CRITERIA	3

TYPE OF VENUE	4

WATER POLO POOL - GENERAL DIMENSIONS	4

FIELD OF PLAY	4

WATER TEMPERATURE	7

LIGHT INTENSITY	7

CEILING HEIGHT FOR INDOOR POOLS	7

ADDITIONAL COMMENTS	8

SIGNATURES	8

# DRAWING SUBMITTAL REQUIREMENTS

As part of the pool certification document, the organizing body is required to submit drawings of the pool and associated competition areas for review in addendum with this document. These drawings must be in scale and reflect the as-built condition of the facility. The drawings must include:

## A building plan view, including the FULL deck widths and spectator seating areas;

## A pool plan view WITH CORRESPONDING WATER DEPTHS;

## A POOL LONGITUDINAL SECTION VIEW WITH CORRESPONDING WATER DEPTHS;

## and A POOL CROSS SECTION VIEW WITH CORRESPONDING WATER DEPTHS.

# MEASUREMENT CRITERIA

## THE MEASUREMENTS MUST BE DONE BY A CERTIFIED SURVEYOR, APPOINTED OR APPROVED BY THE MEMBER IN THE COUNTRY IN WHICH THE VENUE IS LOCATED.

## THE SURVEYING EQUIPMENT MUST BE A TOTAL STATION. THE MINIMUM REQUIREMENTS OF THE TOTAL STATION ARE THE FOLLOWING:

- ANGLE MEASUREMENT ACCURACY HORIZONTAL AND VERTICAL: 1" (0.3 MGON)

- DISTANCE MEASUREMENT ACCURACY: 1MM + 1.5PPM

- THE SURVEYOR MUST PROVIDE A CALIBRATION CERTIFICATE CONFIRMING THE MEASUREMENT QUALITY OF SURVEYING EQUIPMENT. THE CERTIFICATE MUST HAVE A MAXIMUM VALIDITY OF 1 YEAR.

- ALL DIMENSIONS MUST BE MEASURED USING A MINI PRISM OR TARGETS.THE DEPTH OF THE POOL MUST BE MEASURED WITH THE POOL EMPTY.

## ALL THE OTHER MEASUREMENTS MUST BE MEASURED WITH THE POOL FULL OF WATER.

## THE WATER TEMPERATURE MUST NOT BE LESS THAN 26º ± 1º CENTIGRADE PER PART 6, 18.2.4.

# TYPE OF VENUE

The facility must identify the construction material (i.e. Concrete, Stainless Steel, etc.) and pool finish type (i.e. Paint, Plaster, Tile, Liner, etc.).

| POOL CONSTRUCTION |  |
| --- | --- |
| POOL FINISH |  |

# WATER POLO POOL - GENERAL DIMENSIONS

The sides of the pool must be orthogonal and form 90 degrees right angle. The tolerance of the angle is ±0.05º. The 2 diagonals must be the same length. The tolerance of the diagonal is 10mm.

| GENERAL DIMENSIONS |  |
| --- | --- |
| LENGTH 1 [M] |  |
| LENGTH 2 [M] |  |
| WIDTH 1 [M] |  |
| WIDTH 1 [M] |  |
| ANGLE 1 [º] |  |
| ANGLE 2 [º] |  |
| ANGLE 3 [º] |  |
| ANGLE 4 [º] |  |
| DIAGONAL 1 [M] |  |
| DIAGONAL 2 [M] |  |

# FIELD OF PLAY

PART 6, 20.2.3 - The dimensions of the Field of Play are also set out in the diagram at Appendix 1 to this Part Six. However, in the case of any conflict between that Appendix and this Article 20, this Article 20 will prevail.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 6, 20.2.3 |  |  |

PART 6, 20.2 – General Requirements for the Field of Play:

20.2.1 - The limits of the Field of Play are the inner edges of the two (2) Goal Lines and the two (2) Side Lines.

20.2.2.1 - Length: the distance between the inner edge of the two (2) Goal Lines must be 25.00 metres.

20.2.2.2 - Width: the distance between the inner edge of the two (2) Side Lines must be 20.00 metres.

20.2.3 - The dimensions of the Field of Play are also set out in the diagram at Appendix 1 to this Part Six. However, in the case of any conflict between that Appendix and this Article 20, this Article 20 will prevail.

| DIMENSION [M] | MEN/WOMEN |
| --- | --- |
| A1 |  |
| A2 |  |
| A3 |  |
| A4 |  |
| A5 |  |
| B1 |  |
| B2 |  |
| B3 |  |
| B4 |  |
| B5 |  |
| C1 |  |
| C2 |  |
| C3 |  |
| D1 |  |
| D2 |  |
| D3 |  |

PART 6, 20.2.4 - The depth of the water inside the Field of Play will at no point be less than 2.00 metres.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 6, 20.2.4 |  |  |

# WATER TEMPERATURE

PART 6, 20.2.5 - The water temperature must be between 25.0°C and 27.0°C.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 6, 20.2.5 |  |  |

# LIGHT INTENSITY

PART 6, 20.8.2.2 - The light intensity shall not be less than 1500 lux. A lighting report shall be issued in addendum with this document.

|  | YES | NO |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 6, 20.8.2.2 |  |  |

# CEILING HEIGHT FOR INDOOR POOLS

PART 6, 20.2.7 - The Field of Play at the Olympic Games and World Aquatics Championships must have a minimum ceiling height, or minimum height of any arial structure directly above the Field of Play, of not be less than 7.00 metres (measured from the surface of the water).

| CEILING HEIGHT [M] |  |  |
| --- | --- | --- |
| WORLD AQUATICS RULE | COMPLIANT | NON-COMPLIANT |
| PART 6, 20.2.7 |  |  |

# ADDITONAL COMMENTS

This section is provided for additional comments:

# SIGNATURES

| BUILDING NAME |  |  |  |
| --- | --- | --- | --- |
| SURVEYOR |  | SIGNATURE |  |
|  |  | DATE |  |
| POOL OPERATOR |  | SIGNATURE |  |
|  |  | DATE |  |
| ORGANIZING BODY |  | SIGNATURE |  |
|  |  | DATE |  |
| FEDERATION |  | SIGNATURE |  |
|  |  | DATE |  |
| REVIEW STAMP |  |  |  |