ARTISTIC SWIMMING

Pool CERTIFICATION

ORGANIZING BODY

BUILDING NAME

OWNER

CONTRACTOR

DESIGN TEAM

DATE

WORLD AQUATICS

APPROVED BY

DATE

SIGNATURE

In force as from 1 April 2026

Table of Contents

DRAWING SUBMITTAL REQUIREMENTS	3

MEASUREMENT CRITERIA	3

TYPE OF VENUE	4

FACILITY REQUIREMENTS	4

FACILITY REQUIREMENTS FOR OLYMPIC GAMES AND WORLD CHAMPIONSHIPS	6

COMPLIANCE WITH LOCAL REGULATIONS	8

ADDITONAL COMMENTS	9

SIGNATURES	9

# DRAWING SUBMITTAL REQUIREMENTS

As part of the pool certification document, the organizing body is required to submit drawings of the pool and associated competition areas for review in addendum with this document. These drawings must be in scale and reflect the as-built condition of the facility. The drawings must include:

## A building plan view, including the FULL deck widths and spectator seating areas;

## A pool plan view WITH CORRESPONDING WATER DEPTHS;

## A POOL LONGITUDINAL SECTION VIEW WITH CORRESPONDING WATER DEPTHS;

## A POOL CROSS SECTION VIEW WITH CORRESPONDING WATER DEPTHS.

# MEASUREMENT CRITERIA

## THE MEASUREMENTS SHALL BE DONE BY A CERTIFIED SURVEYOR, APPOINTED OR APPROVED BY THE MEMBER IN THE COUNTRY IN WHICH THE VENUE IS LOCATED.

## The surveying equipment shall be a Total Station. The minimum requirements of the Total Station are the following:

- Distance Measurement Accuracy: 1mm + 1.5ppm

## The surveyor shall provide a Calibration Certificate confirming the measurement quality of surveying equipment. The certificate SHALL have a maximum validity of 1 year.

## All dimensions shall be measured using a mini prism or targets.

- The measurements corresponding to THE POOL DEPTH shall be measured with the pool empty

- The measurements corresponding to LENGTH AND WIDTH MAY be measured with the pool full of water. The water level can be lowered up to 5cm to avoid water reflection when measuring.

- The water temperature shall NOT BE LESS THAN 27º Centigrade.

# TYPE OF VENUE

| POOL CONSTRUCTION (I.E. CONCRETE, STAINLESS STEEL, ETC.) |  |
| --- | --- |
| POOL FINISH (I.E. PAINT, PLASTER, TILE, LINER, ETC.) |  |
| POOL USE CASE (I.E OLYMPIC GAMES, WORLD CHAMPIONSHIPS, WORLD CUP TRAINING) |  |

# FACILITY REQUIREMENTS

PART 7, 12.1.1 – Figure sessions and routine sessions of the competition will be conducted in a pool with a minimum area of 15 metres by 25 metres, within which an area of (at least) 12 metres by 12 metres must have a minimum depth of 3.0 metres. The minimum depth of the remaining area will be no less than 2.0 metres.

PART 7, 12.1.2 – Where the general water depth is more than 2.0 metres, the depth at the pool wall may be 2.0 metres and then slope down to reach the general depth (at minimum 3.0 metres) at 1.2 metres maximum from the pool wall.

|  | Length | Width |  | N/A |
| --- | --- | --- | --- | --- |
| Routine Area – 2M |  |  |  |  |
| Routine Area – 3M |  |  |  |  |
| WORLD AQUATICS RULE | COMPLIANT |  | NONCOMPLIANT |  |
| PART 7, 12.1.1 |  |  |  |  |
| PART 7, 12.1.2 |  |  |  |  |

PART 7, 12.1.3 – If the pool bottom does not feature the Swimming lane markings described at Article 15 of Part Two, the pool bottom must be marked with contrasting lines in one direction following the length of the pool as shown in the diagram at Appendix 10.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.3 |  |  |

PART 7, 12.1.4 - The water must be of sufficient clarity for the bottom of the pool to be visible, and the water temperature must not be less than 27° C.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.4 |  |  |
| WATER TEMPERATURE |  |  |

PART 7, 12.1.5 - Measured at the level of 1 metre above the water surface, the minimum light intensity must not be less than 600 lux. Sources of natural and artificial illumination must be provided with controls to prevent glare affecting the Judge’s platforms and the starting platform.

Lighting must be provided at the time of competition. Facilities need only attest that the ability to provide the described level of lighting Is possible. Lighting levels will be confirmed In the Event Certification at the time of the Event.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.5 |  |  |

PART 7, 12.1.6 - The starting platform should be 1.0 metres in height and must not be lower in height than 0.5 metres. The surface of the platform must be covered in a slip-resistant material suggest a quick drying waterproof carpet.

PART 7, 12.1.6 - The Athlete’s entrance should always be centre-stage.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.6 |  |  |
| PART 7, 12.1.7 |  |  |

# FACILITY REQUIREMENTS FOR OLYMPIC GAMES AND WORLD CHAMPIONSHIPS

If the pool will not be used for World Championships or Olympic Games, this section is not required.

PART 7, 12.2.1 – The field of play will be as shown in the diagrams at Appendix 10 and Appendix 11.

PART 7, 12.2.2 – The routine sessions of the competition will be conducted in a pool with a minimum area of 30 metres by 20 metres and a minimum depth of 3.0 metres.

|  | Length | Width |  | N/A |
| --- | --- | --- | --- | --- |
| Routine Area – 3M |  |  |  |  |
| WORLD AQUATICS RULE | COMPLIANT |  | NONCOMPLIANT |  |
| PART 7, 12.2.1 |  |  |  |  |
| PART 7, 12.2.2 |  |  |  |  |

PART 7, 12.2.3 - The light intensity at the level of 1 metre above the water surface shall not be less than 1500lux. Sources of natural and artificial illumination shall be provided with controls to prevent glare for judging platforms and the starting platform.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| PART 7, 12.2.3 |  |  |

PART 7, 12.2.4 - The starting platform should be 1.0 metres in height and must not be lower in height than 0.7 metres (with a tolerance of ±1cm). The surface of the platform must be covered in a slip-resistant material suggest a quick drying water proof carpet.

PART 7, 12.2.5 - The Technical Officials’ platform must be of a minimum height of 0.6 metres and have tables and chairs positioned on it. The platforms must be no more than 2.0 metres from the edge of the pool, as shown in the diagrams at Appendix 10 and Appendix 11.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.2.4 |  |  |
| PART 7, 12.2.5 |  |  |

PART 7, 12.2.6 - The practice and warm up pool must have a minimum area of 25 metres by 25 metres (or 30 metres by 20 metres) with a minimum depth of 3 metres.

PART 7, 12.2.7 - A sound reproduction system that meets the requirements of Article 12.4 of this Part Seven must be available.

PART 7, 12.2.8 - A dry land training and stretching area with mats must be provided for Athletes

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.2.6 |  |  |
| PART 7, 12.2.7 |  |  |
| PART 7, 12.2.8 |  |  |

PART 7, 12.4 - At the Olympic Games, the World Aquatics Championships, the World Aquatics Junior Artistic Swimming Championships, the World Aquatics Youth Artistic Swimming Championships, and the Artistic Swimming World Cup, the sound equipment and presentation standards must include (and at all other international competitions, the sound equipment and presentation standards should include), at minimum:

PART 7, 12.4.1 - An amplifier-mixer system that has at least 16 inputs and 6 outputs, an LR (left-right channels) PA, an LR speaker system on the field of play, and two (2) outputs spare or for broadcasting. Amplifiers should be suitable for the speakers that are used.

PART 7, 12.4.2 - A sound reproduction system [as defined by 12.4.2.1 - 12.4.2.6].

PART 7, 12.4.3 - A public address system that reproduces sound for spectators [as defined by 12.4.3.1 - 12.4.3.4].

PART 7, 12.4.4 - The following additional equipment: a sound volume (decibel) meter for monitoring music sound levels both above and under water; patch cords for interconnecting equipment properly and speaker extension lines adequate for placing speakers for optimal sound distribution; fusing systems as needed to protect speakers and other equipment; grounding lines to ensure safe grounding of all equipment; safety materials to minimise potential of injury to people or equipment from stepping on or tripping over electrical or speaker lines; a stopwatch; tools and meters as needed for initial special hookups and emergency repairs; systems for communication between Technical Officials and the sound desk (five (5) headsets must be available); and a system for continuously monitoring and recording underwater sound.

The sound equipment and presentation standards must be provided at the time of competition. Facilities need only attest that the ability to provide the described systems Is possible. Sound equipment and presentation standards levels will be confirmed In the Event Certification at the time of the Event.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.4.1 |  |  |
| PART 7, 12.4.2 |  |  |
| PART 7, 12.4.3 |  |  |
| PART 7, 12.4.4 |  |  |

# COMPLIANCE WITH LOCAL REGULATIONS

All facilities must comply with local, regional, and national health and safety regulations.

If local regulation require closure for particular conditions, please disclose those in the additional comments section.

| WORLD AQUATICS RULE | YES | NO | N/A |
| --- | --- | --- | --- |
| LOCAL REGULATIONS |  |  |  |
| REGIONAL REGULATIONS |  |  |  |
| NATIONAL REGULATIONS |  |  |  |

# ADDITONAL COMMENTS

This section is provided for additional comments:

# SIGNATURES

| BUILDING NAME |  |  |  |
| --- | --- | --- | --- |
| SURVEYOR |  | SIGNATURE |  |
|  |  | DATE |  |
| POOL OPERATOR |  | SIGNATURE |  |
|  |  | DATE |  |
| ORGANIZING BODY |  | SIGNATURE |  |
|  |  | DATE |  |
| FEDERATION |  | SIGNATURE |  |
|  |  | DATE |  |
| REVIEW STAMP |  |  |  |