ARTISTIC SWIMMING

EVENT CERTIFICATION

ORGANIZING BODY

BUILDING NAME

OWNER

CONTRACTOR

DESIGN TEAM

DATE

WORLD AQUATICS

APPROVED BY

DATE

SIGNATURE

In force as from 1 APRIL 2026

Table of Contents

TYPE OF VENUE	3

FACILITY REQUIREMENTS	3

FACILITY REQUIREMENTS FOR OLYMPIC GAMES AND WORLD CHAMPIONSHIPS	4

COMPLIANCE WITH LOCAL REGULATIONS	6

ADDITONAL COMMENTS	7

SIGNATURES	7

# TYPE OF VENUE

| POOL USE CASE FOR EVENT (I.E OLYMPIC GAMES, WORLD CHAMPIONSHIPS, WORLD CUP TRAINING) |  |
| --- | --- |

# FACILITY REQUIREMENTS

PART 7, 12.1.4 - The water must be of sufficient clarity for the bottom of the pool to be visible, and the water temperature must not be less than 27° C.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.4 |  |  |
| WATER TEMPERATURE |  |  |

PART 7, 12.1.5 - Measured at the level of 1 metre above the water surface, the minimum light intensity must not be less than 600 lux. Sources of natural and artificial illumination must be provided with controls to prevent glare affecting the Judge’s platforms and the starting platform.

|  | YES | NO |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 7, 12.1.5 |  |  |

PART 7, 12.1.6 - The starting platform should be 1.0 metres in height and must not be lower in height than 0.5 metres. The surface of the platform must be covered in a slip-resistant material suggest a quick drying waterproof carpet.

PART 7, 12.1.6 - The Athlete’s entrance should always be centre-stage.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.1.6 |  |  |
| PART 7, 12.1.7 |  |  |

# FACILITY REQUIREMENTS FOR OLYMPIC GAMES AND WORLD CHAMPIONSHIPS

If the pool will not be used for the above-mentioned competitions, this section is not required.

PART 7, 12.2.3 - The light intensity at the level of 1 metre above the water surface shall not be less than 1500lux. Sources of natural and artificial illumination shall be provided with controls to prevent glare for judging platforms and the starting platform.

|  | YES | NO |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 7, 12.2.3 |  |  |

PART 7, 12.2.4 - The starting platform should be 1.0 metres in height and must not be lower in height than 0.7 metres (with a tolerance of ±1cm). The surface of the platform must be covered in a slip-resistant material suggest a quick drying water proof carpet.

PART 7, 12.2.5 - The Technical Officials’ platform must be of a minimum height of 0.6 metres and have tables and chairs positioned on it. The platforms must be no more than 2.0 metres from the edge of the pool, as shown in the diagrams at Appendix 10 and Appendix 11.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.2.4 |  |  |
| PART 7, 12.2.5 |  |  |

PART 7, 12.2.6 - The practice and warm up pool must have a minimum area of 25 metres by 25 metres (or 30 metres by 20 metres) with a minimum depth of 3 metres.

PART 7, 12.2.7 - A sound reproduction system that meets the requirements of Article 12.4 of this Part Seven must be available.

PART 7, 12.2.8 - A dry land training and stretching area with mats must be provided for Athletes

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 7, 12.2.6 |  |  |
| PART 7, 12.2.7 |  |  |
| PART 7, 12.2.8 |  |  |

PART 7, 12.4 - At the Olympic Games, the World Aquatics Championships, the World Aquatics Junior Artistic Swimming Championships, the World Aquatics Youth Artistic Swimming Championships, and the Artistic Swimming World Cup, the sound equipment and presentation standards must include (and at all other international competitions, the sound equipment and presentation standards should include), at minimum:

PART 7, 12.4.1 - An amplifier-mixer system that has at least 16 inputs and 6 outputs, an LR (left-right channels) PA, an LR speaker system on the field of play, and two (2) outputs spare or for broadcasting. Amplifiers should be suitable for the speakers that are used.

PART 7, 12.4.2 - A sound reproduction system [as defined by 12.4.2.1 - 12.4.2.6].

PART 7, 12.4.3 - A public address system that reproduces sound for spectators [as defined by 12.4.3.1 - 12.4.3.4].

PART 7, 12.4.4 - The following additional equipment: a sound volume (decibel) meter for monitoring music sound levels both above and under water; patch cords for interconnecting equipment properly and speaker extension lines adequate for placing speakers for optimal sound distribution; fusing systems as needed to protect speakers and other equipment; grounding lines to ensure safe grounding of all equipment; safety materials to minimise potential of injury to people or equipment from stepping on or tripping over electrical or speaker lines; a stopwatch; tools and meters as needed for initial special hookups and emergency repairs; systems for communication between Technical Officials and the sound desk (five (5) headsets must be available); and a system for continuously monitoring and recording underwater sound.

Documentation of the systems must be provided along with this document.

|  | YES | NO |
| --- | --- | --- |
| DOCUMENTATION PROVIDED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 7, 12.4.1 |  |  |
| PART 7, 12.4.2 |  |  |
| PART 7, 12.4.3 |  |  |
| PART 7, 12.4.4 |  |  |

# COMPLIANCE WITH LOCAL REGULATIONS

All facilities must comply with local, regional, and national health and safety regulations.

If local regulation require closure for particular conditions, please disclose those in the additional comments section.

| WORLD AQUATICS RULE | YES | NO | N/A |
| --- | --- | --- | --- |
| LOCAL REGULATIONS |  |  |  |
| REGIONAL REGULATIONS |  |  |  |
| NATIONAL REGULATIONS |  |  |  |

# ADDITONAL COMMENTS

This section is provided for additional comments:

# SIGNATURES

| BUILDING NAME |  |  |  |
| --- | --- | --- | --- |
| SURVEYOR |  | SIGNATURE |  |
|  |  | DATE |  |
| POOL OPERATOR |  | SIGNATURE |  |
|  |  | DATE |  |
| ORGANIZING BODY |  | SIGNATURE |  |
|  |  | DATE |  |
| FEDERATION |  | SIGNATURE |  |
|  |  | DATE |  |
| REVIEW STAMP |  |  |  |