DIVING

EVENT CERTIFICATION

ORGANIZING BODY

BUILDING NAME

OWNER

CONTRACTOR

DESIGN TEAM

DATE

WORLD AQUATICS

APPROVED BY

DATE

SIGNATURE

In force as from 1 April 2026

# MEASUREMENT CRITERIA

- Diving measurements shall be taken with the pool full of water.

- The water temperature MUST BE NOT less than 28 DEGREES CELCIUS PER PART 4, 15.2.12.1.

# SPRINGBOARD DIVING

PART 4, 15.3.2 - Springboards must be 4.88 metres long and 0.5 metre wide. Springboards must have a slip-resistant surface, which must be inspected and approved by World Aquatics prior to the start of the competition.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.2 |  |  |

PART 4, 15.3.3 - Springboards must be provided with movable fulcrums which can easily be adjusted by Athletes. In addition, for springboards at Diving facilities modified or constructed on concrete platforms after 1st October 2013, the following requirements apply:

15.3.3.1 - The vertical distance from the level of the platform which supports the fulcrum assembly, to the level of the top of the springboard, must be 0.35 metres.

15.3.3.2 - The distance from the front edge of the fulcrum assembly (which must be 0.741 metres in length) to the front edge of the supporting platform, must be a maximum of 0.44 metre. If that requirement is not met, then the fulcrum assembly and the rear hinge assembly must be moved forward so as to provide for a maximum of 0.44 metres from the front edge of the platform to the front of the fulcrum assembly.

15.3.3.3 - The concrete platform that supports the springboard must be aligned with the pool wall or project over the pool.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.3 |  |  |

PART 4, 15.3.4 - The minimum distance from the rear to the centre line of the fulcrum must be set in accordance with the recommendation or specification of the springboard manufacturer.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.4 |  |  |

PART 4, 15.3.5 - A pool must have at least two springboards installed at the same, 3 metre, height, to accommodate synchronised events. The springboards may be placed on either one or both sides of the platform. At least two springboards at the same (3 metre) height must be placed side by side and no objects may obstruct the visibility in any part of the dive between the Athletes (as shown in Appendix 1 and Appendix 2).

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.5 |  |  |

PART 4, 15.3.6 - The springboards must be installed exactly level horizontally, so that their leading edges are exactly level no matter the position of the movable fulcrum.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.6 |  |  |

PART 4, 15.3.7 - The back of the 3 metre springboards must be surrounded (on three sides) by handrails with a minimum clearance of 1.0 metres between vertical pairs. The minimum height for those handrails is 1.0 metre, measured from the level of the springboard. Handrails must be accompanied by at least two horizontal crossbars placed outside the platform, or a solid transparent barrier (see Appendix 1).

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.3.7 |  |  |

# PLATFORM DIVING

PART 4, 15.4.2 - Each platform must be rigid and horizontal.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.4.2 |  |  |

PART 4, 15.4.7 - The surface and the front edge of the platform must be covered throughout with a resilient slip-resistant material. The two surfaces must be covered separately in order to achieve a clean 90 degree angle or as described in Article 15.2.7 of this Part Four. The front surface should be applied first and then the top surface.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.4.7 |  |  |

PART 4, 15.4.8 - The platforms must be covered in a slip-resistant material that has a tread pattern that provides sufficient traction in wet and dry conditions such that the Athletes are prevented from slipping when performing dives in all directions. The minimum thickness of the slip-resistant material is 6mm and the colour must give a contrast to the surrounding décor. The material must be easily cleaned to maintain its slip-resistance.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.4.8 |  |  |

PART 4, 15.4.12 - Each platform must be accessible by suitable stairs (not ladders) as required by the country's building regulations and or health and safety standards that are applicable.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.4.12 |  |  |

# GENERAL REQUIREMENTS

PART 4, 15.1.2.1 - For diving facilities constructed after 1 January 2025, the pool should if possible have a minimum depth of 5 metres in the area of the pool under the springboards and platforms, but in any event must meet the minimum depth(s) set out for the relevant events, in accordance with Appendix 2.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.1.2.1 |  |  |

PART 4, 15.1.2.2 - For diving facilities constructed after 26 September 2013, the minimum dimensions for diving facilities detailed in Appendix 1 and Appendix 2 must be met, using, as a basic measuring point of reference, the plummet line.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.1.2.2 |  |  |

PART 4, 15.2.10 - Mechanical surface agitation must be installed under the diving facilities to aid the Athletes in their visual perception of the surface of the water. A horizontal water sprinkler system must be used for this purpose.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.2.10 |  |  |

PART 4, 15.2.11.2 - At other Diving events held under these Competition Regulations, the minimum illumination at a level of 1 metre above the water surface must not be less than 600 lux throughout the pool.

|  | YES | NO |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 4, 15.2.11.2 |  |  |

PART 4, 15.2.11.3 - Sources of natural and artificial illumination must be provided with controls to prevent glare.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.2.11.3 |  |  |

PART 4, 15.2.12.1 - The water temperature must be not less than 28 degrees Celsius.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| --- | --- | --- |
| PART 4, 15.2.12.1 |  |  |

# DIVING FACILITIES FOR OLYMPIC GAMES AND WORLD CHAMPIONSHIPS

PART 4, 15.6.1 - All Diving facilities and equipment at World Aquatics Events and the Olympic Games, including springboards, platforms and dryland equipment, must comply with the requirements in Article 15 of this Part Four and be inspected and approved by World Aquatics no later than 120 days prior to the start of the competition in accordance with Article 15 of this Part Four.

| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT | N/A |
| --- | --- | --- | --- |
| PART 2, 15.6.1 |  |  |  |

PART 4, 15.6.2.1 - The light intensity at the level of 1m above the water surface must not be less than 1500 lux.

|  | YES | NO |
| --- | --- | --- |
| LIGHTING REPORT ISSUED |  |  |
| WORLD AQUATICS RULE | COMPLIANT | NONCOMPLIANT |
| PART 4, 15.6.2.1 |  |  |

# ADDITONAL COMMENTS

This section is provided for additional comments:

# SIGNATURES

| BUILDING NAME |  |  |  |
| --- | --- | --- | --- |
| SURVEYOR |  | SIGNATURE |  |
|  |  | DATE |  |
| POOL OPERATOR |  | SIGNATURE |  |
|  |  | DATE |  |
| ORGANIZING BODY |  | SIGNATURE |  |
|  |  | DATE |  |
| FEDERATION |  | SIGNATURE |  |
|  |  | DATE |  |
| REVIEW STAMP |  |  |  |